Nerve-Spike Detection Pipeline

Computed Metrics — Meaning & Cross-Condition Comparison Strategy

Source: detectSortNerveSpikesECAP.m  ·  batch_spike_detect.m

Prepared for Arjun  ·  May 2026

# Contents

# 1. What a “run” actually does

Each run takes one blanked recording (a .mat file containing yOut, the multi-channel voltage matrix, and fs, the sampling rate) and pushes every nerve channel through the same pipeline. batch_spike_detect.m is only an orchestrator: it splits stim_rec files into a stim portion and a recovery portion, sends the recovery slice to detectSortNerveSpikesECAP.m, and at the end writes one summary row per file per channel. All the actual measurement happens inside detectSortNerveSpikesECAP.m.

The per-channel sequence is:

- Bandpass filter the channel at 300–5000 Hz (zero-phase filtfilt, 3rd-order Butterworth) to isolate spike-band energy.

- Estimate the noise floor robustly as sigma = median(|signal|) / 0.6745 over “quiet” samples (excludes NaN-blanked and edge-buffer regions).

- Detect candidate peaks above ThreshSigma × sigma (default 6σ), enforcing a refractory minimum spacing.

- Extract and screen waveforms — reject by amplitude bounds (8–150 µV), half-max width bounds (0.15–1.5 ms), and a maximum amplitude cap (40σ) that removes artifacts.

- Quantify the accepted spike train: count, rate, amplitude, SNR, width, ISI quality.

- Firing-rate time series in valid-data-corrected bins, then optional burst detection on the smoothed envelope.

- Spike sorting (PCA + k-means) into putative units, each re-characterised.

- ECAPs — if stimulus times exist, average stimulus-locked snippets into an evoked compound action potential.

Everything is saved three ways: a per-channel <label>_results.mat, a combined detectSortNerveSpikesECAP_allResults.mat, and — at the batch level — detectsort_summary.csv / .mat. The CSV is the one built for cross-file comparison.

Key idea. Most metrics are computed per channel, per run. The summary CSV flattens them to one row per (dataset × channel). Cross-condition analysis is therefore a matter of grouping those rows by a condition label and comparing the right columns the right way — covered in Section 8.

# 2. Detection & signal-conditioning quantities

These define the detector’s operating point. They are not usually reported as results, but they govern every downstream count — and they are the single biggest hidden confound when comparing conditions (see Section 8).

| Metric | Units / field | What it conveys |
| --- | --- | --- |
| Bandpass band | 300–5000 Hz | Frequency window kept before detection. Removes LFP/EMG drift below 300 Hz and noise above 5 kHz. Fixed across runs, so not a variable — but it sets which fiber populations are even visible. |
| sigma (noise) | V, internal | Robust noise estimate, median(¦x¦)/0.6745, over quiet samples. The MAD-based estimator is insensitive to the spikes themselves. This is the scale everything is measured against. |
| thr / thrMax | V, internal | Detection threshold (6σ) and rejection ceiling (40σ). A peak must clear thr to count and stay under thrMax to survive. Because thr is tied to per-run noise, a noisier run is implicitly less sensitive. |
| invalidMask | logical / sample | Samples that are NaN (blanked motion/stim artifact). Excluded from detection and from the firing-rate denominator. |
| edgeMask | logical / sample | Samples within EdgeBufferMs (5 ms) of a blanked segment. Excluded so filter ringing at blank boundaries is not mistaken for spikes. |
| validDuration_s | s | Total recording time that is neither invalid nor edge-masked. The honest denominator for rate: it is what meanRate is divided by, so two runs of different lengths or different blanking remain comparable. |

# 3. Spike & waveform metrics

Computed on the set of accepted spikes (peaks that passed amplitude, width, and refractory screening). These describe how many spikes there are and what they look like.

| Metric | Units / field | What it conveys |
| --- | --- | --- |
| nSpikes | count | Number of accepted spikes in the run. Raw activity volume — but length-dependent, so prefer meanRate for comparisons. |
| meanRate_spk_per_s | spikes/s | nSpikes / validDuration_s. The headline activity metric — average firing rate corrected for blanked time. This is the primary physiological readout for most condition comparisons. |
| spikeTimes / spikeCenters | s / sample | Time of each spike. The raw event train underlying ISI, firing-rate, raster, and burst analyses. |
| waveforms | V matrix | Per-spike snippet (PreMs 0.6 ms before to PostMs 1.0 ms after the peak). The substrate for sorting and shape stats. |
| meanWaveform / stdWaveform | V | Average spike shape and its trial-to-trial spread. Narrow std relative to mean = a clean, consistent unit. |
| Vpp_uv | µV | Peak-to-peak voltage of the mean waveform. Signal magnitude / proximity of the fiber population to the electrode. Sensitive to electrode–nerve coupling, so treat as a recording property, not purely physiology. |
| peakAmps_uv | µV / spike | Per-spike peak-to-peak amplitude. Its distribution shows whether one or several amplitude populations are present. |
| widths_ms (mean/median) | ms | Spike width at half-maximum. A shape descriptor that tracks fiber type / conduction class: narrower spikes tend to be faster, larger-diameter fibers. meanWidth_ms and medianWidth_ms are both in the summary; median is the robust one. |
| SNR | ratio | Vpp / (2 × noiseStd) — mean-spike peak-to-peak over the peak-to-peak of the noise. Detection quality, not a biological effect. Higher = cleaner. Use it to gate which channels are trustworthy before comparing them. |
| isi_ms | ms | Inter-spike intervals (diff(spikeTimes)). Input to refractory-violation and burst structure. |
| refracViolFrac | fraction 0–1 | Fraction of ISIs shorter than the refractory period (1.5 ms). A contamination / sorting-quality gauge: high values mean the spike train mixes multiple units or includes noise. Should be near zero for a clean single unit. |

# 4. Firing-rate time series

computeMaskedFiringRate bins the spike train and divides each bin’s count by its valid (non-masked) seconds, so blanked stretches do not artificially depress the rate.

| Metric | Units / field | What it conveys |
| --- | --- | --- |
| fr_t | s | Bin-centre time axis (default 5 s bins) for the rate series. |
| fr_raw | spikes/s | Per-bin firing rate, count divided by valid seconds in that bin. The instantaneous activity trace; reveals onset, adaptation, recovery dynamics within a run. |
| fr_smooth | spikes/s | Moving-average of fr_raw (default 5 s window). The trace used for plotting and burst detection; less spiky, shows trends. |
| validFrac | fraction 0–1 | Fraction of each bin that was valid data. A confidence weight: bins near 1 are reliable; low-validFrac bins rest on little data and should be down-weighted or dropped in comparisons. |

# 5. Burst metrics

Burst analysis runs only when a channel is active enough (≥500 spikes and mean rate ≥ 0.5 spikes/s); otherwise an empty burst struct is returned. It works on a doubly-smoothed firing-rate envelope and finds prominent peaks.

| Metric | Units / field | What it conveys |
| --- | --- | --- |
| burst.hasBurst | logical | Whether burst structure was detected at all. Also a flag that the channel cleared the activity gate — useful as an inclusion criterion. |
| burst.rate_per_min | bursts/min | Mean burst frequency, mean(1/inter-burst interval) × 60. Captures rhythmic/episodic firing (e.g. respiratory- or migrating-motor-complex-like patterns in vagal recordings). |
| burst.peak_times | s | Times of the firing-rate envelope peaks (burst centres). |
| burst.interval_sec | s | Inter-burst intervals. Their spread shows how regular the bursting is. |
| burst.fr_env | spikes/s | The smoothed envelope on which peaks were found — the visual basis for the burst plot. |

# 6. Spike-sorting (cluster) metrics

When enabled and there are enough waveforms, sortSpikesSimple runs PCA on the waveforms, keeps the top PCs (default 3), and k-means clusters them (default 3 clusters). Clusters smaller than MinClusterSize (10) are discarded to unassigned (id 0). Each surviving cluster is a putative unit and gets its own copy of the spike metrics.

| Metric | Units / field | What it conveys |
| --- | --- | --- |
| clusterID | int / spike | Cluster assignment per spike (0 = unassigned). Lets you split any downstream metric by unit. |
| clusterInfo.nSpikes | count | Spikes in the unit. Relative cluster sizes show which units dominate the activity. |
| clusterInfo.meanWaveform / Vpp_uv | V / µV | The unit’s template shape and amplitude — used to interpret what fiber/unit it is and to match units across runs. |
| clusterInfo.meanAmp_uv | µV | Mean per-spike amplitude within the unit. |
| clusterInfo.refracViolFrac | fraction | Per-unit contamination check. A clean, well-isolated unit has a low value; high means the cluster is not a single unit. |
| clusterInfo.meanRate_spk_per_s / fr_* / burst | spikes/s, etc. | The Section 4–5 metrics recomputed for that unit alone — so you can compare units, not just channels. |

Caveat. NumClusters is fixed at 3 and k-means is seeded (rng(1)) for reproducibility, so the number of “units” is imposed, not discovered. Treat clusters as a decomposition aid; lean on refracViolFrac and waveform separation to judge whether a cluster is real before comparing units across conditions.

# 7. ECAP (evoked compound action potential) metrics

Computed only when stimulus times are available (passed in, derived from a stim vector, or — for stim files — taken from blanked-segment centres). computeECAPs averages stimulus-locked snippets, keeping the artifact window as NaN so the blank does not corrupt the average, and measures the response after the artifact window.

| Metric | Units / field | What it conveys |
| --- | --- | --- |
| ecap.nTriggers | count | Number of stimulus pulses that contributed to the average. The n behind the ECAP — more triggers = more reliable mean and tighter SEM. |
| ecap.mean / std / sem | V | Trial-averaged evoked waveform and its dispersion. The ECAP itself; SEM bands show how well-defined it is. |
| ecap.Vpp_uv | µV | Peak-to-peak of the mean ECAP after the artifact. Evoked response magnitude — how strongly stimulation recruits the nerve. The main stim-efficacy metric. |
| ecap.peakLatency_ms / troughLatency_ms | ms | Times of the post-artifact max and min of the mean ECAP. Latency maps to conduction velocity (distance / latency), so it separates fast (A-fiber) from slow (B/C-fiber) recruitment and is the basis for fiber-selectivity claims. |
| ecap.t_ms / artifactWindow_ms / traces | ms / V | Time axis, the excluded artifact bounds, and the individual trials behind the average. |

# 8. Combining the metrics across conditions

The pipeline already emits the right object for this: detectsort_summary.csv, one row per (dataset × channel), with nSpikes, meanRate_spk_per_s, Vpp_uv, SNR, refracViolFrac, meanWidth_ms, medianWidth_ms, validDuration_s. The work is turning that flat table into condition-aware comparisons without being tripped up by the confounds the pipeline bakes in. Below is a design, not code.

## 8.1 Define the condition axis explicitly

Decide what “condition” means and encode it as columns, parsed from dataset or added by hand. Common axes here:

- State: stim vs recovery vs baseline (the split already separates stim/recovery files).

- Nerve / channel: RVN vs LVN — a within-animal, paired contrast.

- Subject: animal ID — the grouping/random-effect variable.

- Intervention: stim amplitude, frequency, drug, time-point. Pull these from a filename convention or a small metadata sheet you join on dataset.

Goal: every summary row carries animal, channel, condition, and any dose/parameter, so the table can be grouped and pivoted cleanly.

## 8.2 Choose the right metric for the right question

| Metric | Units / field | What it conveys |
| --- | --- | --- |
| “Did activity change?” | meanRate_spk_per_s | Primary outcome. Length- and blanking-corrected, so directly comparable. Use bursts.rate_per_min alongside if the question is about pattern, not level. |
| “Did the evoked response change?” | ecap.Vpp_uv, latencies | Stim-efficacy and fiber-recruitment. Compare ECAP magnitude vs dose; compare latency to argue fiber selectivity. |
| “Which fibers / units?” | median width, cluster templates | Shape-based. Shifts in width distribution or cluster proportions indicate a change in the active fiber population. |
| “Can I trust this channel?” | SNR, refracViolFrac, validDuration_s | Quality gates, not outcomes. Use them to include/exclude before testing — never as the dependent variable in a physiology claim. |

## 8.3 Gate on quality before you compare

Apply uniform inclusion thresholds across all conditions so quality differences don’t masquerade as effects:

- Drop channels with SNR below a fixed cut (e.g. < 2–3) and refracViolFrac above a fixed cut (e.g. > 0.02–0.05).

- Require a minimum validDuration_s so short or heavily-blanked runs don’t enter with noisy rate estimates.

- For ECAPs, require a minimum nTriggers. Record how many channels each condition lost — asymmetric dropout is itself a finding.

## 8.4 Normalise to remove the built-in confounds

Most important caveat. The detection threshold is 6σ of per-run noise. A condition that changes the noise floor (different impedance, movement, electrode drift) changes detection sensitivity — so a raw meanRate difference can be a noise-floor artifact, not a neural one. Vpp_uv and SNR are likewise dominated by electrode–nerve coupling. Normalise before claiming biology.

- Work within electrode/animal. Express each condition relative to that channel’s own baseline: percent change or log fold-change of meanRate, Δ of ECAP Vpp. This cancels per-electrode coupling and per-animal offsets.

- Pair, don’t pool. RVN-vs-LVN and pre-vs-post are paired within animal — keep the pairing rather than averaging everyone together.

- Report the noise floor as a covariate. Carry sigma (or SNR) alongside rate; if rate changes track noise changes, flag the confound explicitly.

- Rate, never count. Always compare meanRate_spk_per_s, never nSpikes, since validDuration_s varies.

## 8.5 Statistics that respect the design

- Paired, two conditions, within animal (e.g. stim vs recovery): Wilcoxon signed-rank (rates are skewed and non-normal); report median Δ and effect size, not just p.

- Multiple animals, unbalanced channels: a linear mixed-effects model with animal (and channel nested in animal) as random effects, condition as fixed effect. This is the honest way to handle several channels per animal without pseudoreplication.

- Dose / parameter sweeps: fit ECAP Vpp or meanRate vs amplitude/frequency per channel, then compare fitted slopes or thresholds across conditions.

- Distributions, not just means: compare full peakAmps_uv / widths_ms / isi_ms distributions with a KS test when the question is about fiber population or firing pattern shifts.

## 8.6 Build one comparison-ready table

Reshape the long summary into an analysis matrix: one row per animal × channel, columns for each metric × condition plus the derived deltas (e.g. meanRate_stim, meanRate_recovery, Δrate_pct). This pivot is what feeds both the stats and the paired plots, and it makes missing cells (dropped channels) visible at a glance. Append ECAP and burst metrics as extra columns from the per-channel result files, joined on dataset + label.

## 8.7 Visualisations that make the comparison legible

- Grouped box / violin of `meanRate` by condition, points coloured by animal — shows effect size and spread together.

- Paired “spaghetti” lines (one line per channel across conditions) — the clearest way to show a paired within-animal effect and who responds.

- Overlaid firing-rate time series aligned to the stim/recovery boundary — reveals onset and recovery dynamics the scalar meanRate hides.

- ECAP overlays across conditions / doses on a common time axis, with SEM bands — magnitude and latency shifts read directly.

- Dose–response curves of ECAP Vpp (or rate) vs amplitude, one curve per condition.

- Confound scatter: `meanRate` vs `SNR` (or `sigma`), coloured by condition — a quick visual check that an apparent rate effect isn’t riding on a noise-floor shift.

- Width / amplitude distribution overlays by condition — evidence for a change in active fiber population.

## 8.8 Suggested workflow, end to end

- Run the batch → collect every detectsort_summary.csv into one master long table.

- Join condition metadata (animal, nerve, state, dose) on dataset/label.

- Apply uniform quality gates; log per-condition inclusion counts.

- Normalise within electrode/animal (fold-change or Δ vs baseline).

- Pivot to the animal × channel comparison matrix; pull in ECAP/burst columns.

- Run paired / mixed-effects stats on the chosen outcome metric.

- Plot paired lines + grouped distributions + the confound scatter; report effect sizes with the p-values.

— End of report —